AGR Hangzhou (启飞智能)

FPV configurator component spec · items and characteristics

All prices in CNY (¥) · USD→CNY rate: 6.84 · FOB multiplier: ×1.10 · Fiber logistics X = 215 CNY/km

Tariff columns

Sample — 1 pc

1,000+ — from 1,000 pcs

5,000+ — from 5,000 pcs

Formulas: USD→CNY = round(USD × rate / fobK); fiber price = round(X × km × multiplier), where multiplier = [1.20, 1.00, 0.98] for [Sample, 1k, 5k].

1. Drone Models

F10 — 10″

Dimensions: 380×310×60 mm · weight w/o battery: 950 g

FC / ESC: F405 · 60A

Motor / prop: 3115 900KV · 10×5×3

VTX: 5.8G 3W · RX: ELRS 915

Battery: 6S 8,000 mAh · 1.2 kg

Video range: 10 km · max speed: 140 km/h

Payload: nominal 2.5 kg / max 3 kg

Flight time: 38 min unloaded · 11 min hover at 2.5 kg

| Version | Subtitle | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- |
| Standard RF | 5.8G radio channel · ELRS 915 included | 1,480 | 1,180 | 1,080 |
| Fiber-optic | Built-in fiber-optic module | 1,280 | 980 | 880 |

F13 — 13″

Dimensions: 445×445×30 mm · weight w/o battery: 1,500 g

FC / ESC: F405 · 80A

Motor / prop: 4214 440KV · 13×10×3

VTX: 5.8G 4W · RX: ELRS 915

Battery: 8S 12,000 mAh · 2.1 kg

Video range: 15 km · max speed: 150 km/h

Payload: nominal 4 kg / max 5 kg

Flight time: 40 min unloaded · 10 min hover at 5 kg

| Version | Subtitle | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- |
| Standard RF | 5.8G radio channel · ELRS 915 included | 2,000 | 1,700 | 1,600 |
| Fiber-optic | Built-in fiber-optic module | 1,700 | 1,400 | 1,300 |

F15 — 15″

Dimensions: 485×485×30 mm · weight w/o battery: 1,670 g

FC / ESC: F405 · 100A

Motor / prop: 4315 375KV · 15×7×3

VTX: 5.8G 4W · RX: ELRS 915

Battery: 8S 16,000 mAh · 2.6 kg

Video range: 15 km · max speed: 170 km/h

Payload: nominal 7 kg / max 8 kg

Flight time: 60 min unloaded · 10 min hover at 8 kg

| Version | Subtitle | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- |
| Standard RF | 5.8G radio channel · ELRS 915 included | 2,100 | 1,900 | 1,800 |
| Fiber-optic | Built-in fiber-optic module | 1,900 | 1,600 | 1,500 |

Common camera characteristics: F1.0 · 1200TVL · Super HDR · starlight night vision.

AI module (option): vehicle detection 1,200 m · person 500 m · up to 50 targets · NPU 6T · trajectory and feature tracking.

MARK4 V2 — F10 configuration with upgrades: FC F722 PRO V2 S8 30.5×30.5 Betaflight + ESC 65A 4-in-1 6S + 1200TVL StarLight HDR camera + 1050×3 prop.

2. Battery (per model)

| ID | Name | Description | Sample | 1,000+ | 5,000+ | Available for |
| --- | --- | --- | --- | --- | --- | --- |
| bat6 | 6S 8,000 mAh | 1.2 kg · for F10 | 350 | 300 | 280 | F10 (default) |
| bat8_12 | 8S 12,000 mAh | 2.1 kg · for F13 | 580 | 530 | 510 | F13 (default) |
| bat8_16 | 8S 16,000 mAh | 2.6 kg · for F15 | 750 | 680 | 650 | F15 (default) |

3. ESC

| ID | Name | Description | Sample | 1,000+ | 5,000+ | Available for |
| --- | --- | --- | --- | --- | --- | --- |
| esc60 | 60A 4-in-1 (6S) | stock | TBD | TBD | TBD | F10 (default) |
| esc65 | 65A 4-in-1 (6S) | upgrade for F10 V2 builds | TBD | TBD | TBD | F10 (option, MARK4 V2) |
| esc80 | 80A 4-in-1 (6S) | stock / upgrade | 245 | 200 | 160 | F10 (option), F13 (default) |
| esc100 | 100A 4-in-1 (8S) | stock | 300 | 230 | 210 | F15 (default) |

4. Flight Controller (FC)

F10 only. On F13/F15 the F405 FC is included in the drone price.

| ID | Name | Description | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- | --- |
| fc_std | F405 (stock) | included in drone price | incl. | incl. | incl. |
| fc_f722 | F722 PRO V2 S8 30.5×30.5 Betaflight | $26 / $23 (computed via rate ÷ fobK) | ≈162 | ≈143 | ≈143 |

5. FPV Camera

Radio buttons. Cameras tagged includesVtx hide the VTX section. cam_dji_o4 additionally blocks AI modules (blocksAi).

| ID | Name | Description | Sample | 1,000+ | 5,000+ | Flags |
| --- | --- | --- | --- | --- | --- | --- |
| cam_std | 1200TVL F1.0 (stock) | Super HDR · night vision | incl. | incl. | incl. | default |
| cam_std_hdr | 1200TVL StarLight HDR | stock variant for MARK4 V2 | incl. | incl. | incl. | — |
| cam_n2 | N2 Pro (night) | outperforms Ratel 2 | 160 | 135 | 120 | — |
| cam_n2p | N2 Pro+ (night+) | outperforms Ratel Pro | 180 | 150 | 135 | — |
| cam_hik_ir | HIK infrared camera | $60 (≈¥373) | dyn. | dyn. | dyn. | — |
| cam_dji_o4 | DJI O4 Air Unit Pro | camera + TX, incompatible with AI · $208 (≈¥1,293) | dyn. | dyn. | dyn. | includesVtx, blocksAi |
| q4max_opt_d | Q4Max (optical) — drone module | anti-jammer, optical, drone module only | 2,150 | 2,150 | 2,150 | includesVtx |
| q4max_opt_dg | Q4Max (optical) — drone + ground | anti-jammer, optical, drone + ground | 2,850 | 2,850 | 2,850 | includesVtx |
| q4max_ir_d | Q4Max (IR) — drone module | anti-jammer, IR, drone module only | TBD | TBD | TBD | includesVtx |
| q4max_ir_dg | Q4Max (IR) — drone + ground | anti-jammer, IR, drone + ground | TBD | TBD | TBD | includesVtx |

6. Video Transmitter (VTX)

Hidden in UI if a camera with includesVtx is selected.

| ID | Name | Description | Price | Available for |
| --- | --- | --- | --- | --- |
| vtx_rf3 | 5.8G 3W VTX (stock) | included in standard version | incl. | F10 (default) |
| vtx_rf4 | 5.8G 4W VTX 48CH MMCX (stock) | included in standard version | incl. | F13/F15 (default) |

7. Receiver (RX)

| ID | Name | Description | Sample | 1,000+ | 5,000+ | Available for |
| --- | --- | --- | --- | --- | --- | --- |
| rx_std | ELRS 915MHz (stock) | included in standard version | incl. | incl. | incl. | all (default) |
| rx_gem | BAYCKRC Gemini C3 900/2400MHz | dual-band, T-shaped 220mm antennas × 2 included, $26.5 (≈¥165) | dyn. | dyn. | dyn. | F10 (option) |

8. TX Module (F10 only)

| ID | Name | Description | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- | --- |
| tx_none | Not needed | transmitter module is built-in | incl. | incl. | incl. |
| tx_nano | BAYCKRC NANO GEMINI ELRS | dual TX 900/2400MHz 1W · Nano→Micro-JR adapter · SMA×2 | 326 | 326 | 326 |

9. Ground Station (Fiber)

| ID | Name | Description | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- | --- |
| fib_no | No ground station | — | incl. | incl. | incl. |
| gnd40 | Ground Station 40 km | reinforced, with battery | 330 | 300 | 270 |
| gnd80 | Ground Station 80 km | reinforced | 400 | 370 | 340 |

10. Fiber Length

Dynamic pricing: round(X × km × multiplier) with X=215 and multipliers [1.20, 1.00, 0.98].

| ID | Length | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- |
| fib_0 | No fiber (RF) | incl. | incl. | incl. |
| fib_5 | 5 km | 1,290 | 1,075 | 1,054 |
| fib_10 | 10 km | 2,580 | 2,150 | 2,107 |
| fib_20 | 20 km | 5,160 | 4,300 | 4,214 |
| fib_30 | 30 km | 7,740 | 6,450 | 6,321 |

11. AI Module (option)

Radio buttons. All options blocked if cam_dji_o4 is selected.

| ID | Name | Description | Sample | 1,000+ | 5,000+ | Tag |
| --- | --- | --- | --- | --- | --- | --- |
| ai_no | No AI | — | incl. | incl. | incl. | — |
| ai1 | Basic — single optics | vehicle 500 m · person 500 m · 50 targets · 6T | 1,300 | 1,200 | 1,100 | AI |
| ai2 | Extended — dual optics | vehicle 1,200 m · person 500 m · 50 targets · 6T | 1,850 | 1,800 | 1,750 | AI |
| ai3 | Thermal 384 — optics + thermal | single optics + thermal 384 | 3,250 | 3,150 | 3,000 | AI |
| ai4 | Pre-top — dual + thermal 384 | dual + thermal 384 | 4,250 | 4,050 | 3,900 | AI |
| ai5 | TOP ★ — dual + thermal 640 | dual + thermal 640 · maximum | 5,200 | 5,100 | 4,800 | TOP★ |

12. Ground Equipment

| ID | Name | Description | Sample | 1,000+ | 5,000+ |
| --- | --- | --- | --- | --- | --- |
| rc | RadioMaster TX12 Mark II controller | ELRS FCC · EdgeTX | 650 | 600 | 580 |
| goggb | Basic 5.8G FPV goggles | standard goggles | 420 | 400 | 380 |
| gogbc | SKYZONE Cobra X V4 | $220 (≈¥1,368) · DVR · Diversity · Steadyview · 5.8G 48CH | dyn. | dyn. | dyn. |
| sd32 | MicroSDHC 32GB V10 A1 | for FPV goggles · × 1 pc/kit | 70 | 70 | 70 |
| chr6 | 6S Charger | HOTA D6 Pro · for F10 | 350 | 300 | 280 |
| chr8 | 8S Charger | for F13 / F15 | 550 | 500 | 450 |
| nano_jr | Nano-JR → Micro-JR Adapter | for TX NANO GEMINI ELRS | 33 | 33 | 33 |

13. Antennas

Same price across all tariffs.

| ID | Name | Description | Price |
| --- | --- | --- | --- |
| ant_mush | Mushroom 5.8G LHCP/RHCP 3.7dBi | circular, onboard | 42 |
| ant_patch | Foxeer Echo Patch 5.8G 8dBi LHCP | directional, ground | 108 |
| ant_moxon9 | Moxon 900/915 MHz LP BETAFPV V2 | directional 900 MHz for ELRS | 48 |
| ant_moxon24 | Moxon 2400 MHz LP BETAFPV V2 | directional 2.4 GHz for ELRS | 41 |
| ant_maple | Maple 5.8G 11dBi LHCP | circular, long-range | 170 |
| ant_helic | Helical 5.8G 13.8dBi LHCP/RHCP | helical, long-range | 67 |
| ant_clover | Clover 5.8G LHCP/RHCP | circular 5.8G | 62 |
| ant_feed | RG-402 SMA male-female feeder | feeder with SMA connectors | 46 |

Legend

incl. — item included in the drone base price / standard version.

TBD — price to be determined.

dyn. — price computed from USD at current rate or by X CNY/km.

includesVtx — component has built-in video TX, VTX section is hidden.

blocksAi — selecting this camera blocks all AI modules except ai_no.

Tags: AI, V2 (improved), TOP★ (top option).

Appendix A. Detailed ESC Specifications

Source: AGR Hangzhou FPV accessory catalogs.

BLS 60A 4-in-1 (stock for F10)

ESC parameters

| ESC size | 45.5 × 41.5 mm |
| --- | --- |
| Board thickness | 1.6 mm |
| Net weight | 12.3 g |
| Mounting holes | 30.5 × 30.5 mm · Ø 4 mm |
| PCB color | matte black |
| Input voltage | 3–6S LiPo |
| Standby current | < 15 mA |
| Continuous current | 60 A (good heat dissipation) |
| Peak current | 80 A (good heat dissipation) |
| BEC | no |
| TVS protection diodes | yes |
| Current sensor ratio | 100 |

Electronics and firmware

| Main MCU | EFM8BB51F16G |
| --- | --- |
| Driver | SA6288 |
| MOSFET | SMT4001AHP |
| Firmware | BLHeli_S E_X_40 |
| Configurator | BLHeliSuite 16.7.14.9.0.3 |
| Supported protocols | PWM, Oneshot125, Oneshot42, Multishot, Dshot150, Dshot300, Dshot600 |
| Signal header pinout | CUR · NC · M4 · M3 · M2 · M1 · VCC · GND |

BLS 100A 4-in-1 (stock for F15)

ESC parameters

| ESC size | 58.0 × 53.2 mm |
| --- | --- |
| Board thickness | 1.6 mm |
| Net weight | 24.5 g |
| Mounting holes | 30.5 × 30.5 mm · Ø 4 mm |
| PCB color | matte black |
| Input voltage | 3–8S LiPo |
| Standby current | < 15 mA |
| Continuous current | 100 A (good heat dissipation) |
| Peak current | 130 A (good heat dissipation) |
| BEC | no |
| TVS protection diodes | yes |
| Current sensor ratio | 100 |

Electronics and firmware

| Main MCU | EFM8BB51F16G |
| --- | --- |
| Drivers | FD6288 / SA6288 |
| MOSFET | SMT6001BHP |
| Firmware | BLHeli_S E_X_40 |
| Configurator | BLHeliSuite 16.7.14.9.0.3 |
| Supported protocols | PWM, Oneshot125, Oneshot42, Multishot, Dshot150, Dshot300, Dshot600 |
| Signal header pinout | CUR · NC · M4 · M3 · M2 · M1 · VCC · GND |

ESC 65A 4-in-1 6S (MARK4 V2 option for F10)

No detailed datasheet in AGR catalogs. From configurator: 4-in-1, 6S, price — TBD. Datasheet request needed from supplier (continuous/peak current, MOSFET, firmware, dimensions, BEC).

ESC 80A 4-in-1 6S (stock for F13)

No detailed datasheet in catalogs. From configurator: 4-in-1, 6S; price ¥245/¥200/¥160 (Sample/1k+/5k+). By analogy with the BLS line, expected: BLHeli_S, Dshot150–600, no BEC, 30.5×30.5 mm form factor. Confirm with supplier.

ESC Summary Table

| Parameter | 60A | 65A | 80A | 100A |
| --- | --- | --- | --- | --- |
| Continuous current | 60 A | 65 A | 80 A | 100 A |
| Peak current | 80 A | — | — | 130 A |
| Input voltage | 3–6S | 6S | 6S | 3–8S |
| Size | 45.5×41.5 mm | — | — | 58.0×53.2 mm |
| Weight | 12.3 g | — | — | 24.5 g |
| Mounting | 30.5×30.5 mm Ø4 | — | — | 30.5×30.5 mm Ø4 |
| MOSFET | SMT4001AHP | — | — | SMT6001BHP |
| Driver | SA6288 | — | — | FD6288 / SA6288 |
| MCU | EFM8BB51F16G | — | — | EFM8BB51F16G |
| Firmware | BLHeli_S E_X_40 | — | — | BLHeli_S E_X_40 |
| Protocols | PWM, Oneshot125/42, Multishot, Dshot150/300/600 | — | — | same |
| BEC / TVS | no / yes | — | — | no / yes |
| Stock on | F10 | F10 (V2 option) | F13, F10 option | F15 |

Appendix B. Detailed Camera Specifications

N2 PRO — FPV camera (night)

Source: AGR Hangzhou catalog. Housing color — black.

| Model | N2PRO |
| --- | --- |
| Sensor | 1/2.4 inch |
| Aperture | F1.0 |
| Horizontal resolution | 1200TVL |
| Video format | NTSC / PAL |
| Aspect ratio | 16:9 / 4:3 |
| WDR | 120 dB |
| Minimum illuminance | 0.1 LUX |
| Field of view (FOV) | H 125°+5° / H 125°+5° / V 75°+5° |
| Electronic shutter | PAL 1/50–100,000 · NTSC 1/60–100,000 |
| Camera OSD | independent |
| Focusing method | Fixed focus (FF), supports day/night co-focusing |
| Scene mode | Auto / Color / Black & White / External |
| Dimensions | 19 × 19 mm (mount depth 15 mm) |
| Voltage input | DC 5–30 V |
| Weight | 8 g ± 2 g |

N2 PRO+ — FPV camera (Starlight enhanced night)

Housing color — red. Differences vs N2 PRO: Starlight low-light mode.

| Model | N2PRO+ |
| --- | --- |
| Sensor | 1/2.4 inch |
| Aperture | F1.0 |
| Horizontal resolution | 1200TVL |
| Video format | NTSC / PAL |
| Aspect ratio | 16:9 / 4:3 |
| WDR | 100 dB |
| Minimum illuminance | 0.01 LUX (Starlight night vision) |
| Field of view (FOV) | H 125°+5° / H 125°+5° / V 75°+5° |
| Electronic shutter | PAL 1/50–100,000 · NTSC 1/60–100,000 |
| Camera OSD | independent |
| Focusing method | Fixed focus (FF), supports day/night co-focusing |
| Scene mode | Auto / Color / Black & White / External |
| Dimensions | 19 × 19 mm (mount depth 15 mm) |
| Voltage input | DC 5–30 V |
| Weight | 8 g ± 2 g |

Q4 Max — wideband video link (VTX + RBox)

Source: AGR Hangzhou catalog + Q4 Max User Guide V2.1. Kit: VTX (airborne transmitter) + RBox (ground receiver, model C403 Max).

Key features

• «4 TX × 4 RX» dual-antenna design — strong anti-jamming, eliminates blind spots.

• End-to-end latency 45 ms, 1080p60fps / 720p60fps.

• Transmission range up to 80 km.

• 3–7 GHz frequency hopping, automatic optimal channel selection, dedicated channels per drone.

• Replaceable IR lens on the sky-end camera; compatible with white-light, large-aperture and IR thermal cameras.

VTX (airborne) — technical parameters

| Communication frequency | 3–7 GHz |
| --- | --- |
| Transmission distance | 80 km |
| Communication bandwidth | 5 MHz |
| Video resolution | 1080p60fps / 720p60fps |
| End-to-end latency | 45 ms |
| Sky-end image sensor | 1/1.8 inch · 2 MP |
| Sky-end camera FOV | 160° |
| Sky-end camera dimensions | 19 × 19 × 24 mm |
| Transmit power (EIRP) | MAX 33 dBm · FCC <30 dBm · CE <14 dBm · SRRC <20 dBm · MIC <25 dBm |
| Interface | JST 1.0×4 (power) · TF card holder |
| SD card slot | up to 1 TB |
| Mounting hole spacing | 25.5 × 25.5 mm |
| Overall VTX dimensions | 34 × 34 × 18 mm |
| VTX weight | 22.41 g |
| VIN input power | 8–40 V |
| UART TX/RX to FC | 3.3 V level |
| Visible camera interface | MIPI 2-lane or 4-lane |
| IR camera interface | MIPI 1-lane or DVP |
| Debug port | UART (RX0/TX0) + USB (VBUS/DP/DM) — firmware/diagnostics |
| Button | Frequency Pairing |

RBox (C403 Max) ground receiver — technical parameters

| Model | C403 Max |
| --- | --- |
| Communication frequency | 3–7 GHz |
| Communication bandwidth | 5 MHz |
| SD card support | up to 1 TB |
| Power input | 9–30 V · 2 A |
| Type-C port | 1) USB-A → Type-C for firmware upgrade · 2) USB 3.2 Type-C for display (DP) · 3) UART EXP, TTL 3.3 V |
| Fan connector | 5 V / 200 mA |
| Button interface | factory keypad (custom keypad board possible) |

RBox antennas

| Polarization | LHCP |
| --- | --- |
| Operating bandwidth | 3–5 GHz · 5–7 GHz |
| Average gain | 10 dBi |
| Input impedance | 50 Ω |
| Standing wave ratio | ≤ 2.5 |
| Interface | SMA-J |
| Weight | 165 g and 95 g |

HIK HM-TM56-9RG/C — thermal imaging module (HIKMICRO)

Source: HIKMICRO HM-TM56-9RG/C datasheet. Applications: UAV pod integration, smart equipment integration.

Features: high-sensitivity uncooled VOx detector 640×512 · analog (CVBS) signal output · automatic, manual and background calibration · detector anti-burn protection · compact design for integration.

Thermal channel

| Detector type | Uncooled vanadium oxide (VOx) microbolometer |
| --- | --- |
| Maximum image size | 640 × 512 |
| Pixel pitch | 12 μm |
| Response band | 8–14 μm |
| NETD (thermal sensitivity) | ≤ 40 mK (@25 °C, F#1.0) |
| MRAD (spatial resolution) | 1.2 |
| Lens focal length | 9.1 mm |
| Field of view | 48.3° (H) × 38.6° (V), 62.4° (D) |
| Minimum focus distance | 2 m |
| Maximum aperture | F1.0 |
| Frame rate | 25 Hz |

Image processing and display

| Output resolution | 720 × 576 |
| --- | --- |
| Electronic zoom | ×1, ×2, ×4, ×8 |
| Image noise reduction | supported |
| Bad pixel correction | supported |
| Mirror | Off / vertical / horizontal / center |
| Detail enhancement | supported |
| AGC mode | histogram |
| Brightness / contrast adjustment | supported |
| Pseudo-color palettes | 15 modes: White Hot, Black Hot, Fusion 1, Rainbow, Fusion 2, Iron Red 1, Iron Red 2 etc. |
| Output interface | CVBS |

System parameters

| Operating temperature | −40 °C … +80 °C |
| --- | --- |
| Storage temperature | −45 °C … +75 °C |
| Storage humidity | < 90 % RH |
| Vibration | Sine 10 m/s², 3 axes |
| Shock | Half-sine 15 g / 11 ms, 3 axes × 6 directions |
| Dimensions | 37.82 × 20 × 20 mm |
| Weight | ≤ 23 g |
| Power consumption | ≤ 0.8 W |
| Input voltage | DC 6–17 V |
| Ordering model | HM-TM56-9RG/C |

Appendix C. Detailed AI Module Specifications

Source: AGR Hangzhou catalog (FPV accessory module addendum) + configurator (specs.ts).

The AI module line is built around a common AI processing board with different optical channels. Base: single visible camera; extended variants add dual optics (near + far) and/or an integrated thermal channel (384 or 640 pixels). All AI options are blocked when the DJI O4 Air Unit Pro camera is selected.

Purpose: automatic detection, recognition and tracking of targets (default: vehicles and pedestrians), crosshair adsorption locking, picture-in-picture mode.

AI Single Optics Module (base, ai1)

Recognition and tracking

| Default target types | vehicle and pedestrian |
| --- | --- |
| Recognition distance (datasheet) | Car: 450 m · People: 170 m |
| Recognition distance (configurator) | Car: 500 m · People: 500 m |
| Minimum target tracking pixel | 16 × 16 |
| Intelligent tracking | Crosshair adsorption locking |
| Max simultaneously recognized targets | 50 |
| Picture-in-Picture | supported |

AI processing board (common to all variants)

| Main computing power | 2 TOPS (datasheet); configurator advertises NPU 6T for top SKUs — confirm with supplier |
| --- | --- |
| Input voltage | 5–18 V |
| Control protocol | CRSF |
| Board dimensions (L×W×H) | 38 × 45 × 18 mm |
| Supported flight controller firmware | Betaflight / ArduPilot |
| Mounting holes | 33.8 × 40.8 mm |

Visible light channel

| Sensor | 1/2.8 inch |
| --- | --- |
| Focal length | 2.26 mm |
| FOV | 160° low distortion |
| Minimum illumination | 0.0001 LUX |
| Video input format | MIPI |
| Camera size (L×W×H) | 35.5 × 25 / 31 × 20.5 mm |

AI Single Optics + Thermal Module (ai3)

Recognition, AI board and visible channel — identical to the base module. Adds a 384 thermal channel.

Single thermal imaging channel

| Pixel pitch | 8 μm |
| --- | --- |
| Response band | 8–14 μm |
| Focal length | 6 mm |
| Video input format | BT.1120 |
| FOV | 48.7° × 39° |
| Camera size (L×W×H) | 34.9 × 21 × 21 mm |

Note: thermal channel runs alongside the visible camera in picture-in-picture / fusion modes for simultaneous target highlighting in day and thermal views.

AI module variants (configurator)

Radio buttons. All AI variants are blocked when cam_dji_o4 is selected (blocksAi flag).

| ID | Name | Composition | Range | Sample | 1,000+ | 5,000+ | Tag |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ai_no | No AI | — | — | incl. | incl. | incl. | — |
| ai1 | Basic | single visible camera | vehicle 500 m · person 500 m · 50 targets · 6T | 1,300 | 1,200 | 1,100 | AI |
| ai2 | Extended | dual visible cameras | vehicle 1,200 m · person 500 m · 50 targets · 6T | 1,850 | 1,800 | 1,750 | AI |
| ai3 | Thermal 384 | single optics + thermal 384 | datasheet range + thermal channel | 3,250 | 3,150 | 3,000 | AI |
| ai4 | Pre-top | dual + thermal 384 | vehicle 1,200 m + thermal | 4,250 | 4,050 | 3,900 | AI |
| ai5 | TOP ★ | dual + thermal 640 | maximum · thermal 640 | 5,200 | 5,100 | 4,800 | TOP★ |

Note: discrepancy between datasheet (car 450 m, person 170 m) and configurator (car 500 m, person 500 m). Configurator also lists NPU «6T» for all AI variants while the datasheet of the AI board states 2 TOPS. Different board revisions likely exist in the line — confirm with supplier.